AUTOID Q9L SERIES PORTABLE TERMINAL

PRODUCT SPECIFICATION V1.0

- SEUIC Technologies Co., Ltd.

| 编订记录 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 文档版本 | 修订日期 | 历史记录 | 编制人 | 审阅人 | 批准人 |
| V0.1 | 2022-2-9 | First edition of Specification | Weijie.Ma |  |  |
| V1.0 | 2023-2-14 | Update Specification | Weijie.Ma |  |  |

- AUTOID Q9L PERFORMANCE,physics and service,environment parameter

| CPU | - MTK Octa core 8*2.0GHz |
| --- | --- |
| Operating System | - Android 10.0 |
| Memory | - 4+64GB |
| Expansion Slot | - Micro SD Card, 32GB SDHC compatible |
| Interface/Communication | - Support Type-C, support OTG |
| Keypad | - 27 keys |
|  | - LED (buttons with backlight) |
| Display | - 4.0 inch size industrial grade low temperature resistant IPS screen, Capacitive, touchable, dual mode touch-with or without a thick glove/with or without wet hand |
|  | - 800(H)×480(W) (WVGA) |
| Power | - Optional detachable 3.85V rechargeable 5200mAh Li-ion battery (built-in backup li-ion battery of 60mAh) |
|  | - Side Type-C interface, support quick charge, intelligent identification of PC and charger - End interface: support 5V/2V charging |
| Notification | - Sound, Vibrator, LED indicator |
| Audio | - Built-in dual loudspeaker, built-in dual microphone, Type-C support earphone |
| Sensor | - Optical sensor, Proximity sensor, Gravity sensor, Gyroscope, E-compass |
| Dimension | - 160(H)×66.3(W)×16.2(T)mm, (17.1mm tall with battery cover) |
| Weight(including standard battery) | - 250g (battery included, varies by different configurations) |
| Operating Temp | - -20℃ to + 50℃ |
| Storage Temp | - -40℃ to + 70℃ (battery excluded) |
| Humidity | - 5% to 95% RH non-condensing |
| Sealing | - IP67 |
| Drop Specification | - Multiple 5.8 ft./1.8m drops to marble across the operating temperature range |
| Tumble Specification | - 1000 rounds of tumble from 0.5m, equals to 2000 impacts |
| ESD | - ±15kV Air discharge, ±8kV direct discharge |
| VoLTE | - Support VoLTE function |

- AUTOID Q9L DATA CAPTURE OPTIONS parameter

| - Function 1 | - 2D Image Scan Engine (option1-X3PSR) |
| --- | --- |
| Sensor Resolution | - 1280(H)×1080(V) |
| Rotate | - 360° |
| Pitch Angle | - ±60° |
| Skew Tolerance | - ±55° |
| Laser Safety Grade | - Class II |
| Image Frame Rate | - 60fps |
| Scan Depth | - Code128: 7cm(3mil), 73cm(20mil) |
| Code Support | - UPC/EAN, UPC/EAN, Code128, GS1-128, Code 39, Code 32, Code 93, Code11, Matrix 2 of 5, Interleaved 2 of 5, IATA 2 of 5, Industrial2 of 5, Codabar, MSI, Code11, PDF417, MicroPDF417, Data Matrix, Maxicode, QR Code, MicroQR, Aztec etc |
| Light Source System | - Warm light, corss-shape laser aiming |
| Minimum Printing Contrast | - 20% |
| - Function 2 | - 2D Image Scan Engine (option2-X3PHD) |
| Sensor Resolution | - 1080(H)×1280(V) |
| Rotate | - 360° |
| Pitch Angle | - ±60° |
| Skew Tolerance | - ±55° |
| Laser Safety Grade | - Class II |
| Image Frame Rate | - 60fps |
| Scan Depth | - Code128: 9cm(3mil), 38cm(20mil) |
| Code Support | - UPC/EAN, UPC/EAN, Code128, GS1-128, Code 39, Code 32, Code 93, Code11, Matrix 2 of 5, Interleaved 2 of 5, IATA 2 of 5, Industrial2 of 5, Codabar, MSI, Code11, etc - PDF417, MicroPDF417, Data Matrix, Maxicode, QR Code, MicroQR, Aztec, etc |
| Light Source System | - Warm light, cross-shape laser aiming |
| Minimum Printing Contrast | - 20% |
| - Function 3 | - 2D Image Scan Engine (option3-X3PMR) |
| Sensor Resolution | - 1080(H)×1280(V) |
| Rotate | - 360° |
| Pitch Angle | - ±60° |
| Skew Tolerance | - ±55° |
| Laser Safety Grade | - Class II |
| Image Frame Rate | - 60fps |
| Scan Depth | - Code128: 10cm(5mil), 84cm(20mil) |
| Code Support | - UPC/EAN, UPC/EAN, Code128, GS1-128, Code 39, Code 32, Code 93, Code11, Matrix 2 of 5, Interleaved 2 of 5, IATA 2 of 5, Industrial2 of 5, Codabar, MSI, Code11, etc - PDF417, MicroPDF417, Data Matrix, Maxicode, QR Code, MicroQR, Aztec, etc |
| Light Source System | - Warm light, cross-shape laser aiming |
| Minimum Printing Contrast | - 20% |
| - Function 4 | - GPS |
| Positioning System | - GPS, Beidou, GLONASS (Three in one) |
| Frequency | - GPS: L1 (1.575GHz); Beidou: B1 (1.561GHz); GLONASS: L1 (1.602GHZ) |
| Maximum Number of Channels | - 31channels |
| Sensitivity | - -130dB (SNR value as 40dBHz) |
| Accuracy | - 5-10meter (OPEN SKY) |
| - Function 5 | - Camera |
| - Front Camera |  |
| Maximum Resolution | - 2592*1944 (taking photo) |
| Photo Function | - Fixed focus |
| Pixel | - 5 mega |
| - Rear Camera |  |
| Maximum Resolution | - 4224*3136 (taking photo), 1080P@30fps (making video) |
| Photo Function | - Auto-foucs |
| Flashlight | - Support flashlight, can be used for lighting |
| Pixel | - 13 mega |
| - Function 6 | - RFID Read/Write (HF) |
| Standards Support | - Support ISO15693, ISO14443A/B (without encryption protocol), ISO14443A tag with encryption protocol (Mifare one S50, S70 and its compatible cards); Support NFC protocol |
| Operating Frequency | - HF frequency band 13.56MHz |
| Reading Range | - Typical values bigger than 6cm for ISO15693, Typical values bigger than 4cm for ISO14443A, and Typical values bigger than 1cm for ISO14443B varies by different tags |

- AUTOID Q9L WIRELESS DATA COMMUNICATIONS parameter

| - Function 1 | - WLAN WIFI |
| --- | --- |
| Protocol | - IEEE 802.11 ac/a/b/g/n/d/e/h/i/j/k/r/v/w (2.4G/5G dual-frequency WIFI) |
| Frequency Range | - Depends on different countries(regions), 2.4GHz: 2.412GHz~2.472GHz; 5GHz: 5.17GHz~5.825GHz |
| Working Channel | - CH1~CH13, CH34~CH165 |
| Security | - OPEN, WEP, WPA2-PSK (TKIP+AES), TLS, TTLS (MS-CHAP), TTLS (MS-CHAP v2), TTLS (CHAP), TTLSMD5, TTLS-PAP, PEAP-TLS, PEAP (MS-CHAP v2), AES, etc |
| Antenna | - Built-in antenna |
| - Function 2 | - WWAN WCDMA |
| Frequency Band | - European Asian area: - GSM: Quad Band (900, 1800Mhz) - TD-SCDMA: Band34, Band39 - WCDMA: Band1, Band5, Band8 - CDMA 1x /EVDO: BC0 - TDD-LTE: Band34, Band38, Band39, Band40, Band41 - FDD-LTE: Band1, Band3, Band5, Band7, Band8, Band20 |
| - Function 3 | - WPAN Bluetooth |
| Standard | - Bluetooth 5.0 (Support BLE) |
| Power | - Class II |

- AUTOID Q9L THIRD-PARTY APPLICATION DEVELOPMENT SUPPORT

| System Software | - Android 10.0 |
| --- | --- |
| System Programming Environment | - Android Studio |

- AUTOID Q9L PERIPHRAL DEVICES AND ACCESSORIES

| Standard Accessories | - Universal Adapter×1, USB Cable×1, Battery×1, Hand Belt×1 |
| --- | --- |
| Optional Accessories | - Four-Slot Battery Charger, Single Slot Charging Cradle Include Battery, Four-Slot Charging Cradle, Pistol Grip, Touch Pen, Tempered Glass Screen Protector, Device Protect Cover |