| 东集 EUIC S |  |  |  |
| --- | --- | --- | --- |
| Android Scanner Interface Document |  |  |  |
| www.seuic.com |  |  |  |
| Revising index |  |  |  |
| Version | Update date | Revising details | By |
| 1.0 | 16/03/2017 | Create the Chinese version | Tao Luo |
| 1.1 | 31/05/2017 | Add specification about Callback function warning | Tao Luo |
| 1.2 | 13/11/2017 | Correct the explanation about image reading interface | Tao Luo |
| 1.3 | 20/11/2017 | Add introduction about new parameter | Tao Luo |
| 1.4 | 03/04/2018 | Add introduction about new parameters(scan led flashing、aimer on/off) | Tao Luo |

Content

Abstract	3rd

Hardware platform	3rd

Software platform	3rd

Object oriented	3rd

Preparation	3rd

Jar package quote method	3rd

The needed access authorization	6th

Scanner related class introduction	6th

Scanner factory class	6th

Scanner class	6th

Scanner data call back interface class	6th

Image reader callback interface class	7th

Scanner statues callback interface class	7th

Scanner data class	8th

Scan button monitor class	8th

Scan interface introduction	9th

Get scanner context	9th

Open the scanner	10th

Close the scanner	10th

Start scan	11th

Stop scan	11th

Set up scan barcode call back	11th

Activate the scanner	12th

Disable the scanner	13th

Get the barcode parameters	13th

Set the barcode parameters	13th

Get the image after successful scanning	14th

Start reading the iamge	15th

Stop reading the image	15th

Set to call back the read image	15th

Set the scanner status call back	16th

Scan parameters introduction	17th

Barcode setting parameters	17th

Scan status parameters	20th

# Abstract

SEUIC device has built-in scan configuration and the corresponding application for customers to choose. Since some clients are in need of developing their own application based on the SUEIC SDK, therefore, this documents is created to introduce scan SDK and help the users better and flexibly use the products.

## Software platform

SDK is supports Eclipse,Android Studio developing tools.

## Object oriented

It’s designed for the developers who would like to customize scan service in their own apps.

# Preparation

## Jar package quote method

1. First, take Exlipse for example, right click Project- Properties - Java Build Path - Libraries - Add External JARs, select ScannerAPI.jar file.

- Take Android Studio Version：2.2.2 for example, find Project, create a new directory under app named as libsref, put the ScannerAPI under the new directory, right click and select the corresponding Module in Open Module Settings, change to the Dependencies in the right and click the “+”, select File dependency and add ScannerAPI.jar into the file, change scope into provided to delcare the ScannerAPI.jar package would be only used in coding instead of running the operation.

①Put the ScannerAPI.jar package into app/libsref/

②Right click and select the corresponding Module in Open Module Settings, change to the Dependencies in the right and click the “+”, select File dependency and add ScannerAPI.jar package into the file.

③Change scope into provided to delcare the ScannerAPI.jar package would be only used in coding instead of running the operation. Then save the settings.

④ If the setting is effective, there would be the below dependencies under build.gradle file.

## The needed access authorization

In the development based on SEUIC SDK, please add the below code under the application nodes in AndroidManifest.xml

<uses-library android:name="android.scanner.library"/>

Please add the below permission for QR code scan for device D500/D510/D330S

<uses-permission android:name="android.permission.CAMERA"/>

<uses-permission android:name="android.permission.WRITE_EXTERNAL_STORAGE"/>

Notes:

1.Please only activate the scanner once in one application, multiple activation would result in time wasting and detention.

2.The scanner can only be switched off when exit the application, stay activated for all the other situation.

3.The developers are free to refer to test demo, define the action in ScannerService, to receive the scan result analysis via registered dynamic broadcast receiver in scan UI.

# Scanner related class introduction

## Scanner factory class

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | ScannerFactory |
| Specification | Scan factory class, mainly for getting the scanner resource |

## Scanner class

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Specification | The main classes realized for scanning, defining a series of scanning method for scanning. |

## Scanner data call back interface class

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | DecodeInfoCallBack |
| Callback function | onDecodeComplete(DecodeInfo barcodeInfo) |
| Callback function parameters | param1: barcode info, DecodeInfo |
| Callback function return value | void |
| Specification | Scan callback function, the users quote the interface and quote the setDecodeInfoCallBack in scanner class for the barcode information. Note: No UI operation should be allowed in the CallBack method. |

## Image reader callback interface class

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | VideoCallBack |
| Callback function | onVideoCallBack(int width, int height, byte[] img) |
| Callback function parameters | param1:Return to image width param2:Return to image height param3: The returned value data for image（Bitmap format） |
| Callback function return value | True: Success, stop the light emitting and image reading false: Failure, continue reading the image. |
| Specification | Scan callback function, the users quote the interface and setVideoCallBack in Scanner class to get the image data, the function would be quoted in every step for getting every frame image. Any third party related operation should be done here. Note: No UI operation should be allowed in the CallBack method. |

## Scanner statues callback interface class

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | StatusCallBack |
| Callback function | onStatusCallBack(int status, int info) |
| Callback function parameters | param1:Status information 0： Scan success 1： Scan timeout 2： Scan canceled 3： Scan error (have not been implemented) 4： Others (have not been implemented) param2: Other extra info (have not been implemented)) Scan status parameters refer to file:Scan Status Parameters |
| Callback function return value | void |
| Specification | Status callback function, the users quote this interface and the setStatusCallBack in Scanner class to get the scanner status upon updating. Note: No UI operation should be allowed in the CallBack method. |

## Scanner data class

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | DecodeInfo |
| Class members | String barcode: Barcode data String codetype: Barcode typle int length: Barcode length |

## Scan button monitor class

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | ScannerKey |
| Class members | KEY_DOWN: Press the scan button down KEY_UP: Loose the press on scan button |
| Class method | int open() Open the scan monitor, return to 1 for success and -1 for failure. int getKeyEvent() Get the key even, return to 1 for success and -1 for failure. void close() Close the scan monitor |
| Specification | Scan button monitor class |

Example code：

import com.seuic.scanner.Scanner;

import com.seuic.scanner.ScannerKey;

import com.seuic.scanner.ScannerFactory;

public class ScannerApiTest {

……

Runnable runnable = new Runnable() {

@Override

public void run() {

int ret1 = ScannerKey.open();

if (ret1 > -1) {

while (true) {

int ret = ScannerKey.getKeyEvent();

if (ret > -1) {

switch (ret) {

case ScannerKey.KEY_DOWN:

scanner.startScan();

break;

case ScannerKey.KEY_UP:

scanner.stopScan();

break;

}

}

}

}

}

};

……

ScannerKey.close();

}

# Scan interface introduction

All types of scanners are supportable without further interface limits specification.

## Get scanner context

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | ScannerFactory |
| Interface | getScanner |
| Interface parameters | Context context |
| Return value | Scanner example |
| Specification | Get scanner class example |

Example code：

import com.seuic.scanner.ScannerFactory；

import com.seuic.scanner.Scanner;

public class ScannerApiTest {

……

Scanner scanner = ScannerFactory.getScanner(context);

……

}

## Open the scanner

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | open |
| Interface parameters | void |
| Return value | boolean |
| Specification | Return to true, successfully open the scanner, false failed at opening the scanner. Note: Only if it returns to true that the scanner is available for other operations. |

Example code：

import com.seuic.scanner.ScannerFactory；

import com.seuic.scanner.Scanner;

public class ScannerApiTest {

……

Scanner scanner = ScannerFactory.getScanner(context);

Boolean status = scanner.open();

……

}

## Close the scanner

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | close |
| Interface parameters | void |
| Return value | void |
| Specification | Release the scanner resource |

Example code：

import com.seuic.scanner.ScannerFactory；

import com.seuic.scanner.Scanner;

public class ScannerApiTest {

……

Scanner scanner = ScannerFactory.getScanner(context);

Boolean status = scanner.close ();

……

}

## Start scan

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | startScan |
| Interface parameters | void |
| Return value | void |
| Specification | Start scanning: the scanner would activate the laser beam and enter into barcode reading mode. |

## Stop scan

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | stopScan |
| Interface parameters | void |
| Return value | void |
| Specification | Stop the scan, so that the scanner would disable the laser beam and stop reading the barcode |

## Set up scan barcode call back

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner DecodeInfoCallBack DecodeInfo |
| Interface | setDecodeInfoCallBack |
| Interface parameters | param1：DecodeInfoCallBack |
| Return value | void |
| Specification | Set up the callback function after ending the scanning for the user, the function is to get the information, length and type information for the barcode. Please make sure the onDecodeComplete method in DecodeInfoCallBack has been realized before making the setting. |

Example code：

import com.seuic.scanner.ScannerFactory；

import com.seuic.scanner.Scanner;

import com.seuic.scanner.DecodeInfoCallBack;

import com.seuic.scanner.DecodeInfo;

public class ScannerApiTest implements DecodeInfoCallBack {

……

Scanner scanner = ScannerFactory.getScanner(context);

scanner.setDecodeInfoCallBack(this);

scanner.startScan();

…….

scanner.stopScan();

@Override

public void onDecodeComplete(DecodeInfo info) {

……

}

……

}

## Activate the scanner

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | enable |
| Interface parameters | void |
| Return value | void |
| Specification | Enable the scanner |

Example code:

import com.seuic.scanner.ScannerFactory；

import com.seuic.scanner.Scanner;

public class ScannerApiTest {

……

Scanner scanner = ScannerFactory.getScanner(context);

scanner.enable();

……

}

## Disable the scanner

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | disable |
| Interface parameters | void |
| Return value | void |
| Specification | Disable the scanner, after that, the scanner won’t be available for any further operation. |

Example code：

import com.seuic.scanner.ScannerFactory；

import com.seuic.scanner.Scanner;

public class ScannerApiTest {

……

Scanner scanner = ScannerFactory.getScanner(context);

scanner.disable();

……

}

## Get the barcode parameters

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | getParams |
| Interface parameters | Param1: parameter code, for example, get the Scan Angle value should with barcode parameter0x02, other detailed parameters refer to Barcode setting parameters |
| Return value | Int type, barcode parameter present value |
| Specification | To get the present setting value for the parameters in according to the parameter code |

## Set the barcode parameters

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | setParams |
| Interface parameters | Param1: parameter code, for example, get the Scan Angle value should with barcode parameter0x02. Param2: need the set parameters Other detailed parameters refer to Barcode setting parameters |
| Return value | boolean: true for successful setting false fir failed setting |
| Specification | Set up the scan parameters in according to the parameter codes |

Example code：

import com.seuic.scanner.ScannerFactory；

import com.seuic.scanner.Scanner;

public class ScannerApiTest {

……

Scanner scanner = ScannerFactory.getScanner(context);

int value = scanner.getParams(0x02);

……

scanner.setParams(0x02, 1);

}

## Get the image after successful scanning

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | getLastImage |
| Interface parameters | void |
| Return value | byte[] :image statistics, width:832 height:640, image format as bitmap |
| Specification | After finish the successful scanning, activate the interface to get the scan image for decoding Note：the interface only applies to the n6603 in D500P, D510P, D500F, D330P,D700 and D700P |

Example code：

import com.seuic.scanner.ScannerFactory；

import com.seuic.scanner.Scanner;

public class ScannerApiTest {

……

Scanner scanner = ScannerFactory.getScanner(context);

scanner.startScan（）;

……

byte[] image = scanner.getLastImage();

}

## Start reading the iamge

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | startVideo |
| Interface parameters | Param1:int type, set the time out with unit as ms. |
| Return value | 0: onVideoCallback call back function, return successfully would automatically exit without activating the stopVide. 9: Exit for invalid parameters 17: Activate the stopVideo to exit manually 6: failure at exit for getting the image 25: Time out and exit 2：Exit with unknown errors |
| Specification | Start reading the image: scanner starts to have laser exited and enter into image reading mode, as the Choke Function. And it cannot be used in the same time with StartScan, it’s only applicable to the n6603 in D500P, D510P, D500F, D330P,D700 and D700P |

## Stop reading the image

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner |
| Interface | stopVideo |
| Interface parameters | void |
| Return value | void |
| Specification | Scanner laser beam stops and image reading stops. It’s only applicable to the n6603 in D500P, D510P, D500F, D330P,D700 and D700P |

## Set to call back the read image

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner VideoCallBack |
| Interface | setVideoCallBack |
| Interface parameters | param1:VideoCallBack |
| Return value | void |
| Specification | Set the call back function after the user’s scan enter into image reading, the interface only apply to the n6603 in D500P, D510P, D500F, D330P ,D700 and D700P |

Example code：

import com.seuic.scanner.ScannerFactory；

import com.seuic.scanner.Scanner;

import com.seuic.scanner.VideoCallBack;

public class ScannerApiTest implements VideoCallBack {

……

Scanner scanner = ScannerFactory.getScanner(context);

scanner.setVideoCallBack(this);

scanner.startVideo(3000);

……

scanner.stopVideo();

@Override

public boolean onVideoCallBack(int width, int height, byte[] img) {

……

}

……

}

## Set the scanner status call back

| Package name | com.seuic.scanner |
| --- | --- |
| Class name | Scanner StatusCallBack |
| Interface | setStatusCallBack |
| Interface parameters | param1: StatusCallBack interface type |
| Return value | void |
| Specification | Set the callback function for the present scanner status for the user, please make sure the onStatusCallBack in StatusCallBack has been realized before setting up, the related scan status would automatically activate the onStatusCallBack and give the present status feedback to the users. |

Example code：

import com.seuic.scanner.ScannerFactory；

import com.seuic.scanner.Scanner;

public class ScannerApiTest implements StatusCallBack {

……

Scanner scanner = ScannerFactory.getScanner(context);

scanner.setStatusCallBack(this);

@Override

public boolean onStatusCallBack(int status, int info) {

……

}

……

}

# Scan parameters introduction

## Barcode setting parameters

| Parameter | Parameter Number (Hex) |
| --- | --- |
| Laser On Time | 0x01 |
| Scan Angle | 0x02 |
| Time Out Between Same Symbol | 0x03 |
| Linear Code Type Security Levels | 0x04 |
| Bi-directional Redundancy | 0x06 |
| Aimer/Illumination lighting combinations | 0x07 |
| Mobile Phone/Display Mode | 0x08 |
| Illumination Power Level | 0x09 |
| Decode Time Out | 0x0a |
| Adaptive Scanning | 0x0b |
| Continue Scan Mode | 0x0c |
| Exposure Mode | 0x0d |
| Picklist Mode | 0x0e |
| Aim Accuracy Level | 0x0f |
| GS1 AI Separator | 0x10 |
| Transfer Aim ID | 0x11 |
| Inverse Mode | 0x12 |
| Low Power Delay Time | 0x13 |
| Hold Scanning Time Out | 0x14 |
| Set Aimer On/Off | 0x15 |
| UPC-A | 0x101 |
| UPC-E | 0x102 |
| UPC-E1 | 0x103 |
| EAN-8 | 0x104 |
| EAN-13 | 0x105 |
| Bookland EAN | 0x106 |
| Transmit UPC-A Check Digit | 0x107 |
| Transmit UPC-E Check Digit | 0x108 |
| Transmit UPC-E1 Check Digit | 0x109 |
| Transmit EAN-13 Check Digit | 0x10a |
| Transmit EAN-8 Check Digit | 0x10b |
| UPC-A Preamble | 0x10c |
| UPC-E Preamble | 0x10d |
| UPC-E1 Preamble | 0x10e |
| Convert UPC-E to A | 0x10f |
| Convert UPC-E1 to A | 0x110 |
| Convert EAN-8 to 13 | 0x111 |
| UCC Coupon Extended Code | 0x112 |
| UPC-E Expand | 0x113 |
| UPC-A 2 Char Addenda | 0x114 |
| UPC-A 5 Char Addenda | 0x115 |
| UPC-E 2 Char Addenda | 0x116 |
| UPC-E 5 Char Addenda | 0x117 |
| EAN13 2 Char Addenda | 0x118 |
| EAN13 5 Char Addenda | 0x119 |
| EAN8 2 Char Addenda | 0x11a |
| EAN8 5 Char Addenda | 0x11b |
| Code 128 | 0x121 |
| Code 128 Minimum Length | 0x122 |
| Code 128 Maximum Length | 0x123 |
| ISBT 128 | 0x124 |
| GS1-128 | 0x125 |
| ISBT 128 Predefined Concatenation Sequences On/Off | 0x126 |
| Code 39 | 0x131 |
| Trioptic Code 39 | 0x132 |
| Code 39 Full ASCII | 0x133 |
| Convert Code 39 to Code 32 | 0x134 |
| Code 32 Prefix | 0x135 |
| Code 39 Minimum Length | 0x136 |
| Code 39 Maximum Length | 0x137 |
| Transmit Code 39 Check Digit | 0x138 |
| Code 39 Check Digit Verification | 0x139 |
| Transmit Code 39 Start/Stop Characters | 0x13a |
| Code 39 Quiet Zone | 0x13b |
| Code 93 | 0x0141 |
| Code 93 Minimum Length | 0x142 |
| Code 93 Maximum Length | 0x143 |
| Code 11 | 0x151 |
| Code 11 Minimum Length | 0x152 |
| Code 11 Maximum Length | 0x153 |
| Code 11 Check Digit Verification | 0x154 |
| Transmit Code 11 Check Digit | 0x155 |
| Interleaved 2 of 5 | 0x161 |
| Interleaved 2 of 5 Minimum Length | 0x162 |
| Interleaved 2 of 5 Maximum Length | 0x163 |
| Discrete 2 of 5 | 0x171 |
| Discrete 2 of 5 Minimum Length | 0x172 |
| Discrete 2 of 5 Maximum Length | 0x173 |
| Chinese 2 of 5 | 0x181 |
| Chinese 2 of 5 Minimum Length | 0x182 |
| Chinese 2 of 5 Maximum Length | 0x183 |
| Codabar | 0x191 |
| Codabar Minimum Length | 0x192 |
| Codabar Maximum Length | 0x193 |
| Transmit Codabar Start/Stop Characters | 0x194 |
| MSI | 0x201 |
| MSI Minimum Length | 0x202 |
| MSI Maximum Length | 0x203 |
| RSS 14 | 0x211 |
| RSS limit | 0x212 |
| RSS expanded | 0x213 |
| Convert RSS to UPC/EAN | 0x214 |
| QR Code | 0x221 |
| QR Code Minimum Length | 0x222 |
| QR Code Maximum Length | 0x223 |
| QR Inverse | 0x224 |
| MicroQR | 0x225 |
| Data Matrix | 0x231 |
| Data Matrix Minimum Length | 0x232 |
| Data Matrix Maximum Length | 0x233 |
| Data Matrix Inverse | 0x234 |
| Data Matrix Decode Mirror Images | 0x235 |
| China Post | 0x241 |
| China Post Minimum Length | 0x242 |
| China Post Maximum Length | 0x243 |
| Matrix 2 of 5 | 0x251 |
| Matrix 2 of 5 Minimum Length | 0x252 |
| Matrix 2 of 5 Maximum Length | 0x253 |
| US Postnet | 0x261 |
| US Planet | 0x262 |
| British Post | 0x263 |
| Japanese Post | 0x264 |
| Australian Post | 0x265 |
| UPU FICS Postal | 0x266 |
| USPS 4CB/One Code/Intelligent Mail | 0x267 |
| KIX(Netherlands) Post | 0x268 |
| Transmit US Postal Check Digit | 0x269 |
| Transmit Planet Check Digit | 0x26a |
| Korean 2 of 5 | 0x271 |
| EAN UCC Composite | 0x2b1 |
| EAN UCC Composite Minimum Length | 0x2b2 |
| EAN UCC Composite Maximum Length | 0x2b3 |
| PDF417 | 0x2c1 |
| PDF417 Minimum Length | 0x2c2 |
| PDF417 Maximum Length | 0x2c3 |
| Micro PDF417 | 0x2d1 |
| Micro PDF417 Minimum Length | 0x2d2 |
| Micro PDF417 Maximum Length | 0x2d3 |
| MaxiCode | 0x2e1 |
| MaxiCode Minimum Length | 0x2e2 |
| MaxiCode Maximum Length | 0x2e3 |
| Aztec Code | 0x2f1 |
| Aztec Code Minimum Length | 0x2f2 |
| Aztec Code Maximum Length | 0x2f3 |
| Aztec Inverse | 0x2f4 |
| Codablock | 0x301 |
| Codablock Minimum Length | 0x302 |
| Codablock Maximum Length | 0x303 |
| Telepen | 0x311 |
| Telepen Minimum Length | 0x312 |
| Telepen Maximum Length | 0x313 |
| Telepen Old Syle Mode | 0x314 |
| Set Default Parameter | 0x321 |
| Set Code Charset | 0x322 |
| Scan Len Flashing | 0x323 |
| OCR | 0x331 |
| Set OCR Template | 0x332 |
| Composite CC-C | 0x341 |
| Composite CC-A/B | 0x342 |
| Composite TLC-39 | 0x343 |
| UPC Composite | 0x344 |
| Transmit Code ID Character | 0x351 |
| Grid Matrix | 0x361 |

Aimer On/Off

Attention:

- This parameter is only for the controls of Aimer.

- This parameter will not interrupt the operation of startScan, after scanning the aimer will be turned off, we have to set this parameter to control the Aimer for next operation.

Detailed scanner parameters information refer to : Scanner barcode parameter summary (internal only)

## Scan status parameters

| Status | Status Number(Hex) |
| --- | --- |
| Decode Complete | 0x00 |
| Decode Time Out | 0x01 |
| Decode has been Canceled | 0x02 |
| Some Errors Happened | 0x03 |
| Decode Event | 0x04 |