ZK_RFID9816 UHF RFID Reader

User’s Manual v1.10

Content

1. COMMUNICATION INTERFACE SPECIFICATION	1

2. PROTOCOL DESCRIPTION	1

3. DATA BLOCK FORMAT	2

3.1 COMMAND DATA BLOCK	2

3.2 RESPONSE DATA BLOCK	2

4. OPERATION COMMAND (CMD) SUMMARY	3

4.1 EPC C1G2（ISO18000-6C）COMMAND	3

4.2 READER DEFINED COMMAND	4

5. LIST OF COMMAND EXECUTION RESULT STATUS	6

6. TAG RETURNED ERROR CODE	8

7. TAG FEATURES REQUIRING ATTENTION	8

8. DETAILED DESCRIPTION OF OPERATION COMMANDS	9

8.1 OVERVIEW	9

8.2 EPC C1G2 COMMAND	9

8.2.1 Inventory	9

8.2.2 Read Data	11

8.2.3 Write Data	12

8.2.4 Write EPC	14

8.2.5 Kill Tag	14

8.2.6 Lock	15

8.2.7 BlockErase	17

8.2.8 Set Privacy with Mask Pattern	18

8.2.9 Set Privacy without Mask Pattern	19

8.2.10 Reset Privacy	19

8.2.12 EAS Configure	20

8.2.13 EAS Alarm	21

8.2.14 Single Tag Inventory	21

8.2.15 Block Write	22

8.2.16 Read Monza4QT parameters	23

8.2.17 Set Monza4QT parameters	24

8.2.18 Extension Read Data	25

8.2.19 Extension Write Data	27

8.2.20 Buffer Inventory	28

8.3 READER-DEFINED COMMAND	29

8.3.1 Get Reader Information	29

8.3.2 Set Region	31

8.3.3 Set Address	32

8.3.4 Set InventoryScanTime	32

8.3.5 Set Baud Rate	32

8.3.6 Set RF Power	33

8.3.7 Beep Setting	33

8.3.8 Set GPIO	33

8.3.9 Get GPIO Status	34

8.3.10 Get Reader Serial number	34

8.3.11 Set Tag Custom function	35

8.3.12 Set Buffer EPC length	35

8.3.13 Get Buffer EPC length	35

8.3.14 Get Buffer data	36

8.3.15 Clear Buffer	37

8.3.16 Get Buffer tag number.	37

# 1. COMMUNICATION INTERFACE SPECIFICATION

The reader communicates with host (MCU，MPU，Controller) using serial communication interface RS232/ RS485 or TCPIP and complete corresponding operation according to the host command. The default serial communication parameter is 57600bps 1 start bit, 8 data bits, 1 stop bit without parity check bit. In the process of communication, the least significant bit of one byte is transmitted first and the least significant byte of command data sequence is transmitted first.

# 2. PROTOCOL DESCRIPTION

A communication procedure is sponsored by the host sending commands and data to the reader and the reader returns the result status and data to host after command execution.

Reader executes a command after receiving it. Only after command execution completed, the reader would be able to receive another command. During the implementation of one command, the reader ignores all other command data received.

The following table shows the process of the host computer sending command:

| HOST | DIRECTION | READER |
| --- | --- | --- |
| Command Data Block | → |  |

The interval between two consecutive bytes in the command data block should be less than 15ms. During command data block sending, synchronization will lost if the host receives any data from the reader and the host should stop command sending and restart the communication after 15ms.

The reader completes command execution after receiving host command and returns the results. During the period, it doesn’t process any host data. The feedback of command execution results is as follows:

| READER | DIRECTION | HOST |
| --- | --- | --- |
| Response Data Block | → |  |

The interval between two consecutive bytes in the response data block is less than 15ms.

# 3. DATA BLOCK FORMAT

## 3.1 COMMAND DATA BLOCK

| Len | Adr | Cmd | Data[] | LSB-CRC16 | MSB-CRC16 |
| --- | --- | --- | --- | --- | --- |

Remark：

|  | LENGTH(Byte) | COMMENT |
| --- | --- | --- |
| Len | 1 | Command data block length 1 byte (not including itself). Value range is 4~96. The number of Len equals the length of Data [] plus 4. |
| Adr | 1 | Reader address, 1 byte. Value range is 0~254. Only will the reader conforming to the address response the command data block. Value 255 is broadcasting address. All the readers will response to the command data block with a broadcasting address. The default value is 0. |
| Cmd | 1 | Operation command symbol, 1 byte. |
| Data[] | Variable | Operation command parameters. |
| LSB-CRC16 | 1 | CRC-16 LSB. CRC-16 checksum, 2 bytes with least significant byte first. |
| MSB-CRC16 | 1 | CRC-16 MSB. |

## 3.2 RESPONSE DATA BLOCK

| Len | Adr | reCmd | Status | Data[] | LSB-CRC16 | MSB-CRC16 |
| --- | --- | --- | --- | --- | --- | --- |

COMMENT：

|  | LENGTH(Byte) | COMMENT |
| --- | --- | --- |
| Len | 1 | Response data block length 1 byte (not including itself). The number of Len equals the length of Data [] plus 5. |
| Adr | 1 | Reader address, 1 byte. Value rang is 0~254. |
| reCmd | 1 | Received command symbol, 1 byte. If the command is unrecognized, the reCmd is 0x00. |
| Status | 1 | Result status value, 1byte. Refer to following table for details. |
| Data[] | Variable | Response data. |
| LSB-CRC16 | 1 | CRC16 LSB. CRC-16 checksum, 2 bytes with least significant byte first. |
| MSB-CRC16 | 1 | CRC16 MSB. |

The default value of the reader address is 0x00. The host may change it by using reader-defined command “Write Adr”.

Cyclic Redundancy Check (CRC) computation includes all data from Len. A reference CRC computation program is presented as follow:

C-Example:

#define PRESET_VALUE 0xFFFF

#define POLYNOMIAL  0x8408

unsigned int uiCrc16Cal(unsigned char const  * pucY, unsigned char ucX)

{

unsigned char ucI,ucJ;

unsigned short int  uiCrcValue = PRESET_VALUE;

for(ucI = 0; ucI < ucX; ucI++)

{

uiCrcValue = uiCrcValue ^ *(pucY + ucI);

for(ucJ = 0; ucJ < 8; ucJ++)

{

if(uiCrcValue & 0x0001)

{

uiCrcValue = (uiCrcValue >> 1) ^ POLYNOMIAL;

}

else

{

uiCrcValue = (uiCrcValue >> 1);

}

}

}

return uiCrcValue;

}

# 4. OPERATION COMMAND (CMD) SUMMARY

## 4.1 EPC C1G2（ISO18000-6C）COMMAND

| NUM | COMMAND | CODE | COMMENT |
| --- | --- | --- | --- |
| 1 | Inventory | 0x01 | The function is used to inventory tags in the effective field and get their EPC values. |
| 2 | Read Data | 0x02 | The function is used to read part or all of a Tag’s Password, EPC, TID, or User memory. |
| 3 | Write Data | 0x03 | The function is used to write several words in a Tag’s Reserved, EPC, TID, or User memory. |
| 4 | Write EPC | 0x04 | The function is used to write EPC value in one Tag’s EPC memory. The writing process is carried out in a broadcast way. |
| 5 | Kill Tag | 0x05 | The function is used to deactivate (kill) one tag. When killed, the tag will not answer to any command. |
| 6 | Lock | 0x06 | The function is used to set the tag memories’ access control policy by setting them as readable, writable or on the verse. |
| 7 | BlockErase | 0x07 | The function is used to erase multiple words in a Tag’s Password, EPC, TID, or User memory. |
| 8 | Set Privacy with Mask Pattern | 0x08 | The function is used to set a designated tag into privacy state (Only NXP's UCODE EPC G2X tags available). |
| 9 | Set Privacy without Mask Pattern | 0x09 | The function is used to set one tag in the field into privacy state (Only NXP's UCODE EPC G2X tags available). |
| 10 | Reset Privacy | 0x0a | The function is used to remove one tag from privacy state(Only NXP's UCODE EPC G2X tags available). |
| 11 | Check Privacy | 0x0b | The function is used to check one tag if it is in the privacy state (Only NXP's UCODE EPC G2X tags available). |
| 12 | EAS Configure | 0x0c | The function is used to set or reset the EAS bit of one designated tag (Only NXP's UCODE EPC G2X tags available). |
| 13 | EAS Alarm | 0x0d | The function is used to check EAS bit status of one tag in the field (Only NXP's UCODE EPC G2X tags available). |
| 14 | Single Tag Inventory | 0x0f | The function is used to get one tag’s EPC value in the field. |
| 15 | Block Write | 0x10 | The function is used to write multiple words in a Tag’s Reserved, EPC, TID, or User memory. |
| 16 | Read Monza4Qt | 0x11 | The function is used to get Monza4QT work parameters. |
| 17 | Set Monza4Qt | 0x12 | The function is used to Set Monza4QT work parameters. |
| 18 | Extension Read | 0x15 | The function is used to read part or all of a Tag’s Password, EPC, TID, or User memory. |
| 19 | Extension Write | 0x16 | The function is used to write several words in a Tag’s Reserved, EPC, TID, or User memory. |
| 20 | Buffer Inventory | 0x18 | The function is used to get tag count in the buffer |

## 4.2 READER DEFINED COMMAND

| NUM | COMMAND | CODE | CONNECT |
| --- | --- | --- | --- |
| 1 | Get Reader Information | 0x21 | This function is used to get reader-related information such as reader address (Adr), firmware version, supported protocol type, InventoryScanTime, RF power and frequency range. |
| 2 | Set Region | 0x22 | This function is used to set the work region which defines the lower limit and the upper limit of frequency range. |
| 3 | Set Address | 0x24 | This function is used to set the reader’s address. The address value will be stored in reader’s inner nonvolatile memory with default value 0x00. The value range is 0x00~0xFE. The address 0xFF is reserved as the broadcasting address. When user tries to write a 0xFF address, the reader will set the value to 0x00 automatically. |
| 4 | Set InventoryScanTime | 0x25 | This function is used to set reader’s InventoryScanTim. The value range is 3~255 corresponding to 3*100ms~255*100ms with default 10 (10*100ms). |
| 5 | Set Baud Rate | 0x28 | The function is used to change the serial communication baud rate. |
| 6 | Set RF Power | 0x2F | The function is used to set the RF power of reader. |
| 7 | Set GPIO | 0x46 | The function is used to set GPIO status. |
| 8 | Get GPIO Status | 0x47 | The function is used to read GPIO status. |
| 9 | Get seria Number | 0x4C | The function is used to get reader‘s seria number. |
| 10 | Set tag custom Function | 0x3a | The function is used to set tag’s defined function.. |
| 11 | Beep Setting | 0x40 | The function is used to set read’s beep status |
| 12 | Set buffer EPC length | 0x70 | The function is used to set buffer EPC length |
| 13 | Get buffer EPC length | 0x71 | The function is used to get buffer EPC length |
| 14 | Get buffer data | 0x72 | The function is used to get buffer data |
| 15 | Clear buffer | 0x73 | The function is used to clear buffer data |
| 16 | Get tag number | 0x74 | The function is used to get tag ‘s number in the buffer. |

# 5. LIST OF COMMAND EXECUTION RESULT STATUS

| RESPONSE DATA BLOCK |  |  |  |  |  | STATUS COMMENT |
| --- | --- | --- | --- | --- | --- | --- |
| Len | Adr | reCmd | Status | Data[] | CRC16 |  |
| Length of Data[] +5 | 0xXX | 0xXX | 0x00 | …….. | LSB+MSB | Command is executed successfully and data part contains result data. |
| Length of Data[] +5 | 0xXX | 0x01 | 0x01 | …….. | LSB+MSB | G2 tag inventory operation is completely finished in defined InventoryScanTime and all tags’ EPC data are returned. |
| Length of Data[] +5 | 0xXX | 0x01 | 0x02 | …….. | LSB+MSB | G2 tag inventory operation is not completely finished when defined InventoryScanTime overflows and some tags’ EPC data are returned. |
| Length of Data[] +5 | 0xXX | 0x01 | 0x03 | …….. | LSB+MSB | The number of the inventoried tags is too much and not all tags’ EPC data can be returned within this single response data block. Other response data blocks are followed. |
| Length of Data[] +5 | 0xXX | 0x01 | 0x04 | …….. | LSB+MSB | G2 tag inventory operation is finished since the number of the inventoried tags reaches the reader process limit. All inventoried tags’ EPC data are returned. |
| 5 | 0xXX | 0xXX | 0x05 | - | LSB+MSB | The sent command is password-required operation and the password given in the command is wrong. |
| 5 | 0xXX | 0x05 | 0x09 | - | LSB+MSB | Kill command is not executed successfully. The possible reasons include password error and poor reader to tag air communication. |
| 5 | 0xXX | 0x05 | 0x0a | - | LSB+MSB | Kill Password is zero. Kill tag operation requires a none-zero password. |
| 5 | 0xXX | 0xXX | 0x0b | - | LSB+MSB | The operation not supported by the tag. |
| 5 | 0xXX | 0xXX | 0x0c | - | LSB+MSB | Password is zero. Privacy or EAS related operations require a none-zero password (Only NXP's UCODE EPC G2X tags available). |
| 5 | 0xXX | 0x0a | 0x0d | - | LSB+MSB | The tag is already in privacy. It does not need to be set into privacy state again (Only NXP's UCODE EPC G2X tags available). |
| 5 | 0xXX | 0x0a | 0x0e | - | LSB+MSB | The tag is not in privacy state and does not need to be set out of privacy state (Only NXP's UCODE EPC G2X tags available) or the tag does not support this kind of operation. |
| 5 | 0xXX | 0x53 | 0x10 | - | LSB+MSB | The data block of a 6B tag is locked and can not be rewritten. |
| 5 | 0xXX | 0x55 | 0x11 | - | LSB+MSB | The data block of a 6B tag can not be locked. |
| 5 | 0xXX | 0x55 | 0x12 | - | LSB+MSB | The data block of a 6B tag is already locked and not needed to be re-locked. |
| 5 | 0xXX | 0xXX | 0x13 | - | LSB+MSB | Reader configuration parameters saving operation failed. |
| 5 | 0xXX | 0xXX | 0x14 | - | LSB+MSB | The RF power can not be adjusted in current situation. |
| Length of Data[] +5 | 0xXX | 0x51 | 0X15 | …….. | LSB+MSB | 6B tag inventory operation is completely finished in defined InventoryScanTime and all tags data are returned. |
| Length of Data[] +5 | 0xXX | 0x51 | 0x16 | …….. | LSB+MSB | 6B tag inventory operation is not completely finished when defined InventoryScanTime overflows and some tags’ data are returned. |
| Length of Data[] +5 | 0xXX | 0x51 | 0x17 | …….. | LSB+MSB | The number of the inventoried tags is too much and not all tags’ data can be returned within this single response data block. Other response data blocks are followed. |
| Length of Data[] +5 | 0xXX | 0x51 | 0x18 | …….. | LSB+MSB | 6B tag inventory operation is finished since the number of the inventoried tags reaches the reader process limit. All inventoried tags’ data are returned. |
| 5 | 0xXX | 0xXX | 0x19 | - | LSB+MSB | EAS related operation failed. The possible reasons include the tag does not support this operation or the password is zero. |
| 5 | 0xXX | 0xXX | 0xF8 | - | LSB+MSB | Antenna checked error. |
| 5 | 0xXX | 0xXX | 0xF9 | - | LSB+MSB | Command execution error. |
| 5 | 0xXX | 0xXX | 0xFA | - | LSB+MSB | Operation aborted since the poor reader to tag air communication. |
| 5 | 0xXX | 0xXX | 0xFB | - | LSB+MSB | No tag in the field. |
| 6 | 0xXX | 0xXX | 0xFC | Err_code | LSB+MSB | Tag operation error and the error code is returned in Err_code. |
| 5 | 0xXX | 0xXX | 0xFD | - | LSB+MSB | Command length error. |
| 5 | 0xXX | 0x00 | 0xFE | - | LSB+MSB | Command can not be recognized since it is a non-exist command or CRC error. |
| 5 | 0xXX | 0xXX | 0xFF | - | LSB+MSB | Command parameters error. |

# 6. TAG RETURNED ERROR CODE

EPC C1G2（ISO18000-6C）Tag returned error code：

|  | Error-Code | Error-Code Name | Description |
| --- | --- | --- | --- |
| Specified error | 0x00 | Other error | Other errors. |
|  | 0x03 | Memory override | Memory not exist or not-supported PC value. |
|  | 0x04 | Memory locked | Memory is locked and can not be rewritten. |
|  | 0x0b | Insufficient power | No sufficient power to meet writing operation requirement. |
| Non-specified error | 0x0f | Non-specified error | Non-specified error. |

# 7. TAG FEATURES REQUIRING ATTENTION

EPC C1G2 TAG（G2 TAG）

G2 tag memory includes four sector: reserved memory (password memory), EPC memory, TID memory and User memory.

Reserved memory (password memory) is a 4-word sector. The former 2 word is kill password and the latter 2 word is access password. All passwords are available to read and write. Reserved memory can be read/write protected by configuration..

EPC memory contains EPC information of the tag. The address 0 word is CRC checksum of the EPC memory. The address 1 word is PC value indicating the EPC word-unit length. From the address 2 word on, it is the EPC of the tag. EPC memory is available to read and write.

TID memory contains information such as UID etc. provided by the tag vendor. TID memory is available to read and not to write.

User memory is a space for tag user. User memory is available to read and write. User memory is optional.

In G2 tag operation, the word unit is used frequently. 1 word equals 2 bytes.

Some G2 tag operation need password and if the password is not set, please use all 0s instead.

# 8. DETAILED DESCRIPTION OF OPERATION COMMANDS

## 8.1 OVERVIEW

The reader supports three kinds of commands: protocol related command, reader command and tag customized command.

If the reader gets an unrecognized command such as non-exist command or CRC error command, it will return following response data block:

| Len | Adr | reCmd | Status | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x00 | 0xFE | LSB | MSB |

If the reader gets a command with incorrect length, it will return the following response data block:

| Len | Adr | reCmd | Status | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0xXX | 0xFD | LSB | MSB |

The reader will not respond to next two kinds of commands:

1. A command with incorrect address. That means the Adr byte in the command is neither equal to the reader’s address nor to the broadcast address 0xFF;

2. An un-integrated command. That means the length indicated by the Len byte is greater than the actual received command length.

## 8.2 EPC C1G2 COMMAND

### 8.2.1 Inventory

The command is used to inventory tags in the field and get their EPC values. The user may need to use this function to get tags’ EPC before any further operation to the tags.

This command is executed within predefined InventoryScanTime limit. The default InventoryScanTime is 1 second (10*100ms). The allowable value range is from 3*100ms to 255*100ms.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x01 | - | LSB | MSB |

Data as follows:

| QValue | Session | MaskMem | MaskAdr | MaskLen | MaskData |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0xXX | 2Bytes | 0xXX | Variable |

| Data[] |  |  |
| --- | --- | --- |
| Target | Ant | Scantime |
| 0xXX | 0x80 | 0xXX |

Parameter explanation:

QValue: One byte, Q value, range is 0-15. Q value setting should be field label quantity is approximately equal to 2 Q.

Session: One byte, Session value,range is 0-3.

0x00:S0;

0x01:S1;

0x02:S2;

0x03:S3;

MaskMem: One byte. It specifies the target memory when applying inventory mask with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address of the mask pattern data. The value ranges from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern data. The value ranges from 0 to 255.

MaskData: Mask pattern data. The byte length of the MaskData is MaskLen/8. If MaskLen is not 8bits integer times, the length of MaskData should be int[MaskLen/8]+1 with 0 patching in the low significant location.

Target(Optional parameters):One byte,

0x00：Target value is A;

0x01：Target value is B;

Other values are reserved.

Ant(Optional parameters):One byte,0x80, Other values are reserved.

ScanTime(Optional parameters): One byte,reader will set max scan time to ScanTime*100ms.

Remark:

- The MaskMem , MaskAdr, MaskLen and MaskData can be all vacant. That means tags will be inventoried without mask pattern.

- The Target, Ant, ScanTime is optional parameters,these parameters must be used to together.

Response:

| Len | Adr | reCmd | Status | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ant | Num | EPC ID |  |  |
| 0xXX | 0xXX | 0x01 | 0xXX | 0x01 | 0xXX | EPC-1, EPC-2, EPC-3… | LSB | MSB |

Parameter explanation:

Status:

| Value | Comment |
| --- | --- |
| 0x01 | Tag inventory operation is completely finished in defined InventoryScanTime and all tags’ EPC data are returned. |
| 0x02 | Tag inventory operation is not completely finished when defined InventoryScanTime overflows and some tags’ EPC data are returned. |
| 0x03 | The number of the inventoried tags is too much and not all tags’ EPC data can be returned within this single response data block. Other response data blocks are followed. |
| 0x04 | Tag inventory operation is finished since the number of the inventoried tags reaches the reader process limit. All inventoried tags’ EPC data are returned. |

Ant: It describes from which antenna the tag EPC is collected,this module is 0x01.

Num: The number of tag detected.

EPC ID: Inventoried tag’s EPC data. EPC-1 is the first tag’s EPC length plus EPC data plus RSSI and so on. The most significant word of EPC is transmitted first and the most significant byte of a word is also transmitted first. EPC length is one byte.

### 8.2.2 Read Data

The command is used to read part or all of a Tag’s Password, EPC, TID, or User memory.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x02 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Mem | WordPtr | Num | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 0xXX | 0xXX | 0xXX | 4Byte | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: One byte. It specifies the starting word address for the operation with 0x00 for the first word, 0x01 for the second word and so on.

Num: One byte. It specifies the number of words to read. The value range is from 1 to 120. Other value will incur a parameters error.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x02 | 0x00 | Word1，Word2,… | LSB | MSB |

Parameter explanation:

Word1, Word2….: read out data in word units with most significant byte of a word first. Word1 is the content of the start-address-defined word in target memory and so on.

### 8.2.3 Write Data

The command is used to write several words into a Tag’s Reserved, EPC, TID or User memory.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x03 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WNum | ENum | EPC | Mem | WordPtr | Wdt |
| 0xXX | 0xXX | Variable | 0xXX | 0xXX | Variable |
| Pwd | MaskMem | MaskAdr | MaskLen | MaskData |  |
| 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |  |

Parameter explanation:

WNum: One byte. It specifies the number of words to be written into a tag. The value can not be 0 and should be equal to the word length of Wdt.

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: One byte. It specifies the starting word address for the operation with 0x00 for the first word, 0x01 for the second word and so on.

Wdt: the data to be written into a tag. The word length of Wdt should be equal to WNum and the data should be arranged as most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x03 | 0x00 | - | LSB | MSB |

### 8.2.4 Write EPC

The command is used to write EPC code in a Tag’s EPC memory. When using this command to write the EPC code, please make sure that there is only one tag in the effective field since this command utilize no mask pattern for selecting one tag to operate and just take one tag in the field to operate instead.

Command:

| Len | Adr | Cmd | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ENum | Pwd | WEPC |  |  |
| 0xXX | 0xXX | 0x04 | 0xXX | 4Bytes | Variable | LSB | MSB |

Parameter explanation:

ENum: one byte. It specifies the tag’s EPC length in word unit and the value range is from 0 to 15.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

WEPC: The EPC code to be written. The word length of EPC should be equal to Enum and the data should be arranged as most significant word first and most significant byte in a word first.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x04 | 0x00 | - | LSB | MSB |

### 8.2.5 Kill Tag

The command is used to kill tag. After killed, a tag will not answer any request.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x05 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Killpwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Killpwd: Four bytes kill password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed and all 0s kill password is not allowed..

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x05 | 0x00 | - | LSB | MSB |

### 8.2.6 Lock

The command is used to set protection strategy of Reserved, EPC, TID and User memory.

Reserved memory can be set as unprotected readable/writable, permanent readable/writable, password-protected readable/writable and permanent unreadable/unwritable;

EPC and User memory can be set as unprotected writable, permanent writable, password-protected writable and permanent writable. These two memories are always readable;

TID memory is always readable and unwritable.

If the Reserved memory is set to permanent readable/writable or permanent unreadable/unwritable, it can not be set to other state any more.

If EPC or TID memory is set to permanent writable or permanent unwritable, it can not be changed to other state any more.

If a memory is set to password protected state, any access to this memory should be carried out with a correct password.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x06 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Select | SetProtect | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 0xXX | 0xXX | 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Select: One byte. It specifies the operation target as follows:

0x00: Kill Password section. Kill Password section is the first half of the Reserved memory;

0x01: Access Password section. Access Password section is the second half of the Reserved memory;

0x02: EPC memory;

0x03: TID memory;

0x04: User memory.

Other values are reserved.

SetProtect: one byte. It specifies the protection strategy of the Select defined target memory.

When Select is 0x00 or 0x01, SetProtect has the following meaning::

0x00: unprotected readable/writable;.

0x01: permanent readable/writable;

0x02: password-protected readable/writable;

0x03: permanent unreadable/unwritable.

When Select is 0x02, 0x03 or 0x04, SetProtect has the following meaning:

0x00: unprotected writable;.

0x01: permanent writable;

0x02: password-protected writable;

0x03: permanent unwritable.

Other values are reserved.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x06 | 0x00 | - | LSB | MSB |

### 8.2.7 BlockErase

The command is used to erase multiple words in a Tag’s Reserved, EPC, TID or User memory. BlockErase is an advance but non-mandatory feature of EPC C1G2 tag and please check the tag’s datasheet before using it.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x07 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Mem | WordPtr | Num | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 0xXX | 0xXX | 0xXX | 4Byte | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: One byte. It specifies the starting word address for the operation with 0x00 for the first word, 0x01 for the second word and so on. If the target memory is the EPC memory, the WordPtr should not be 0.

Num: One byte. It specifies the number of words to erase.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x07 | 0x00 | - | LSB | MSB |

### 8.2.8 Set Privacy with Mask Pattern

The command is used to set designated tag into privacy state. When a tag is in privacy state, it will ignore common command such as read, write and query etc. This is an advanced feature which is available for NXP’s UCODE EPC G2X tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x08 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first..

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password and the password should not be all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x08 | 0x00 | - | LSB | MSB |

### 8.2.9 Set Privacy without Mask Pattern

The command is used to set a tag in the field into privacy state. When a tag is in privacy state, it will ignore common command such as read, write and query etc. This is an advanced feature which is available for NXP’s UCODE EPC G2X tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Pwd |  |  |
| 0x08 | 0xXX | 0x09 | 4Bytes | LSB | MSB |

Parameter explanation:

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password and the password should not be all 0s.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x09 | 0x00 | - | LSB | MSB |

### 8.2.10 Reset Privacy

The command is used to release a tag from privacy state into normal state. This is an advanced feature which is available for NXP’s UCODE EPC G2X tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Pwd |  |  |
| 0x08 | 0xXX | 0x0a | 4Bytes | LSB | MSB |

Parameter explanation:

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password and the password should not be all 0.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x0a | 0x00 | —— | LSB | MSB |

### 8.2.12 EAS Configure

The function is used to set or reset the EAS bit of a designated tag. This is an advanced feature which is available for NXP’s UCODE EPC G2X tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x0c | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Pwd | EAS | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 4Bytes | 0xXX | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password and the password should not be all 0s.

EAS: One byte. Bit0=0 means reset EAS state, Bit0=1 means set EAS state. Bit1~Bit7 are reserved.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x0c | 0x00 | - | LSB | MSB |

### 8.2.13 EAS Alarm

The function is used to check EAS bit’s status of a tag in the field. This is an advanced feature which is available for NXP’s UCODE EPC G2X tag.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x0d | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x0d | 0x00 | - | LSB | MSB |

If no tag’s EAS bit is set, the response status will be “no tag in the field”.

### 8.2.14 Single Tag Inventory

This command is used to get one tag’s EPC value in the field.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x0f | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ant | Num | EPC ID |  |  |
| 0xXX | 0xXX | 0x0f | 0x01 | 0x01 | 0x01 | EPC-1 | LSB | MSB |

Num: one byte. It specifies the number of tag inventoried and should always be 1 if tag detected.

Ant: It describes from which antenna the tag EPC is collected,this module is 0x01.

EPC ID: Inventoried tag’s EPC data. EPC-1 is the first tag’s EPC length plus EPC data. The most significant word of EPC is transmitted first and the most significant byte of a word is also transmitted first. EPC length is one byte.

### 8.2.15 Block Write

The command is used to write multiple words in a Tag’s Reserved, EPC, TID or User memory. BlockWrite is an advance but non-mandatory feature of EPC C1G2 tag and please check the tag’s datasheet before using it.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x10 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WNum | ENum | EPC | Mem | WordPtr | Wdt |
| 0xXX | 0xXX | Variable | 0xXX | 0xXX | Variable |
| Pwd | MaskMem | MaskAdr | MaskLen | MaskData |  |
| 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |  |

Parameter explanation:

WNum: One byte. It specifies the number of words to be written into a tag. The value can not be 0 and should be equal to the word length of Wdt.

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: One byte. It specifies the starting word address for the operation with 0x00 for the first word, 0x01 for the second word and so on.

Wdt: the data to be written into a tag. The word length of Wdt should be equal to WNum and the data should be arranged as most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x10 | 0x00 | - | LSB | MSB |

### 8.2.16 Read Monza4QT parameters

The command is used to read Monza4Qt work parameters.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x11 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | recmd | Status | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NC | QTcontrol |  |  |
| 0x07 | 0xXX | 0x11 | 0x00 | 0x00 | 1 byte | LSB | MSB |

Parameter explanation:

QTcontrol: Tag work parameters.

Bit0：Mirror page.bit0=0:private;bit0=1:public.

Bit1：Enabled distance protect or not.bit1=0:diabled;bit1=1:enabled.

### 8.2.17 Set Monza4QT parameters

The command is used to Set Monza4Qt work parameters.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x12 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | QTcontrol1 | QTcontrol0 | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 0xXX | 0xXX | 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

QTcontrol1: reserved，0x00;

QTcontrol0: Tag work parameters.

Bit0：mirror page.bit0=0:private;bit0=1:public.

Bit1：enabled distance protect or not.bit1=0:diabled;bit1=1:enabled.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x12 | 0x00 | - | LSB | MSB |

Parameter explanation:

QTcontrol: Tag work parameters.

Bit0：mirror page.bit0=0:private;bit0=1:public.

Bit1：enabled distance protect or not.bit1=0:diabled;bit1=1:enabled.

### 8.2.18 Extension Read Data

The command is used to read part or all of a Tag’s Password, EPC, TID, or User memory.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x15 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENum | EPC | Mem | WordPtr | Num | Pwd | MaskMem | MaskAdr | MaskLen | MaskData |
| 0xXX | Variable | 0xXX | 2Bytes | 0xXX | 4Byte | 0xXX | 2Bytes | 0xXX | Variable |

Parameter explanation:

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: Two byte. It specifies the starting word address for the operation with 0x0000 for the first word, 0x0001 for the second word and so on.

Num: One byte. It specifies the number of words to read. The value range is from 1 to 120. Other value will incur a parameters error.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x15 | 0x00 | Word1，Word2,… | LSB | MSB |

Parameter explanation:

Word1, Word2….: read out data in word units with most significant byte of a word first. Word1 is the content of the start-address-defined word in target memory and so on.

### 8.2.19 Extension Write Data

The command is used to write several words into a Tag’s Reserved, EPC, TID or User memory.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x016 | - | LSB | MSB |

Data as follows:

| Data[] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WNum | ENum | EPC | Mem | WordPtr | Wdt |
| 0xXX | 0xXX | Variable | 0xXX | 2Bytes | Variable |
| Pwd | MaskMem | MaskAdr | MaskLen | MaskData |  |
| 4Bytes | 0xXX | 2Bytes | 0xXX | Variable |  |

Parameter explanation:

WNum: One byte. It specifies the number of words to be written into a tag. The value can not be 0 and should be equal to the word length of Wdt.

ENum: one byte length indicator in word unit (1 word = 2 bytes).

When ENum’s value is in 0~15, it represents the EPC length. And in this case, the MaskMem , MaskAdr, MaskLen and MaskData shall all be absent.

When ENum is 0xff, there shall be MaskMem, MaskAdr, MaskLen and MaskData and no EPC presented. The MaskMem, MaskAdr, MaskLen and MaskData construct a mask pattern for the operation.

Other value for ENum is prohibited and will incur a parameter error.

EPC: tag’s whole EPC code. Its word length is defined by ENum with most significant word first and most significant byte in a word first.

Mem: One byte. It specifies the target memory of the operation with 0x00 for Password memory, 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

WordPtr: Two byte. It specifies the starting word address for the operation with 0x0000 for the first word, 0x0001 for the second word and so on.

Wdt: the data to be written into a tag. The word length of Wdt should be equal to WNum and the data should be arranged as most significant word first and most significant byte in a word first.

Pwd: Four bytes access password. The most significant byte (MSB) of access password is the MSB of Pwd. Please remember to set Pwd the correct password if operation needed or set Pwd all 0s.

MaskMem: One byte. It specifies the target memory when applying mask pattern with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address in target memory when applying mask pattern. The value range is from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern.

MaskData: mask pattern data. The length of MaskData is MaskLen/8. If MaskLen can not be divided exactly by 8, the length of MaskData is int(MaskLen/8)+1 with 0 padding in the least significant bit of last byte of MaskData.

Notes: The operation actually uses the tag’s whole EPC code as the mask pattern when the MaskMem, MaskAdr, MaskLen and MaskData are vacant.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x16 | 0x00 | - | LSB | MSB |

### 8.2.20 Buffer Inventory

This command is used to identify the tabbed operation, at the same time will save tag data in buffer.

Diffent with 8.2.1 "office" command, legibility in predefined office hours continuous plate tag, the tag data stored in the internal cache area (buffer can be set to EPC maximum length 128 - bit or 496 bit two formats), office at the end of the office time arrival, and returns the buffer zone and the office of the total number of tags in tag number (the same tag read will be many times repeatedly counts). Allows users to cache the data extraction, clear the cache and query cache area commands such as tag number to access the data in the cache

This command is executed within predefined InventoryScanTime limit. The default InventoryScanTime is 1 second (10*100ms). The allowable value range is from 3*100ms to 255*100ms.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x18 | - | LSB | MSB |

Data as follows:

| QValue | Session | MaskMem | MaskAdr | MaskLen | MaskData |
| --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0xXX | 2Bytes | 0xXX | Variable |

| Data[] |  |  |
| --- | --- | --- |
| Target | Ant | Scantime |
| 0xXX | 0x80 | 0xXX |

Parameter explanation:

QValue: One byte, Q value, range is 0-15. Q value setting should be field label quantity is approximately equal to 2 Q.

Session: One byte, Session value,range is 0-3.

0x00:S0;

0x01:S1;

0x02:S2;

0x03:S3;

MaskMem: One byte. It specifies the target memory when applying inventory mask with 0x01 for EPC memory, 0x02 for TID memory and 0x03 for User memory. Other values are reserved.

MaskAdr: Two bytes. It specifies the start bit address of the mask pattern data. The value ranges from 0 to 16383.

MaskLen: One byte. It specifies the bit length of the mask pattern data. The value ranges from 0 to 255.

MaskData: Mask pattern data. The byte length of the MaskData is MaskLen/8. If MaskLen is not 8bits integer times, the length of MaskData should be int[MaskLen/8]+1 with 0 patching in the low significant location.

Target(Optional parameters):One byte,

0x00：Target value is A;

0x01：Target value is B;

Other values are reserved.

Ant(Optional parameters):One byte,0x80, Other values are reserved.

ScanTime(Optional parameters): One byte,reader will set max scan time to ScanTime*100ms.

Remark:

- The MaskMem , MaskAdr, MaskLen and MaskData can be all vacant. That means tags will be inventoried without mask pattern.

- The Target, Ant, ScanTime is optional parameters,these parameters must be used to together.

Response:

| Len | Adr | reCmd | Status | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | BufferCount | TagNum |  |  |
| 0x09 | 0xXX | 0x18 | 0x00 | 2Bytes | 2Bytes | LSB | MSB |

Parameter explanation:

BufferCount: 2 bytes, Total number of tag record in buffer, same EPC data tag will be deemed to be the same. If not clear the buffer, tag number for many times the number of office operation accumulation

TagNum: 2 bytes,Read tag times current query, do not distinguish whether read many times with same tag.

## 8.3 READER-DEFINED COMMAND

### 8.3.1 Get Reader Information

The command is used to get the reader’s information such as firmware version, reader type code, supporting protocol, RF power, work frequency band, InventoryScanTime etc.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x21 | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x11 | 0xXX | 0x21 | 0x00 | Version, Type, Tr_Type, dmaxfre, dminfre, Power, Scntm, Ant, Beepen, Reserved, Reserved | LSB | MSB |

Parameter explanation:

| Parameter | Length(Byte) | Connect |
| --- | --- | --- |
| Version | 2 | The first byte specifies the main version and the second byte specifies the subversion. |
| Type | 1 | The reader type code . |
| Tr_Type | 1 | It specifies the protocol the reader supports. two bits (bit1&bit0) are set to 1 with bit1=1 for ISO18000-6C and bit0=1 for ISO18000-6B. |
| dmaxfre | 1 | Bit7~Bit6 is used to indicate frequency band and Bit5~Bit0 is used to specify the maximum frequency point. As to the frequency band definition, please refer to the following table. Please also refer to 8.3.2. |
| dminfre | 1 | Bit7~Bit6 is used to indicate frequency band and Bit5~Bit0 is used to specify the minimum frequency point. As to the frequency band definition, please refer to the following table. Please also refer to 8.3.2. |
| Power | 1 | It specifies the reader’s RF output power. The value range is 0 to 26. |
| Scntm | 1 | It specifies the InventoryScanTime. Please also refer to 8.2.1 |
| Ant | 1 | RFU |
| Beepen | 1 | It specifies the Beep status. |
| Reserved | 1 | RFU |
| CheckAnt | 1 | RFU |

Frequency Band Table:

| dmaxfre(Bit7) | dmaxfre(Bit6) | dminfre(Bit7) | dminfre(Bit6) | Region Frequency Band |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | RFU |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 0 | 1 | RFU |
| … | … | … | … | … |
| 1 | 1 | 1 | 1 | RFU |

### 8.3.2 Set Region

The command is used to set the reader’s working frequency band and the maximum and minimum frequency point in the band.

Command:

| Len | Adr | Cmd | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MaxFre | MinFre |  |  |
| 0x06 | 0xXX | 0x22 | 0xXX | 0xXX | LSB | MSB |

Parameter explanation:

MaxFre: one byte. Bit7~Bit6 is used to indicate frequency band and Bit5~Bit0 is used to specify the maximum frequency point. As to the frequency band definition, please refer to the following table.

MinFre: one byte. Bit7~Bit6 is used to indicate frequency band and Bit5~Bit0 is used to specify the minimum frequency point. As to the frequency band definition, please refer to the following table.

Please note that MaxFre should be greater than MinFre.

Frequency Band Table:

| MaxFre(Bit7) | MaxFre(Bit6) | MinFre(Bit7) | MinFre(Bit6) | Region Frequency Band |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | RFU |
| 0 | 0 | 0 | 1 | Chinese band2 |
| 0 | 0 | 1 | 0 | US band |
| 0 | 0 | 1 | 1 | Korean band |
| 0 | 1 | 0 | 0 | EU band |
| 0 | 1 | 0 | 1 | RFU |
| … | … | … | … | … |
| 1 | 1 | 1 | 1 | RFU |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x22 | 0x00 | - | LSB | MSB |

Various Region Frequency Band Calculation:

Chinese band2:			Fs = 920.125 + N * 0.25 (MHz), N∈ [0, 19].

US band:					Fs = 902.75 + N * 0.5 (MHz), N∈ [0, 49].

Korean band:				Fs = 917.1 + N * 0.2 (MHz), N∈ [0, 31].

EU band:				    Fs = 865.1 + N*0.2(MHz) N∈[0, 14]。

### 8.3.3 Set Address

This command is used to set the reader’s address.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Address |  |  |
| 0x05 | 0xXX | 0x24 | 0xXX | LSB | MSB |

Parameter explanation:

Address: one byte. It specifies the reader’s address and the value range is 0 to 254. 255 is the broadcasting address.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x24 | 0x00 | - | LSB | MSB |

Notes: The Adr in response is the old address and not the new address.

### 8.3.4 Set InventoryScanTime

The command is used to set the InventoryScanTime.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Scantime |  |  |
| 0x05 | 0xXX | 0x25 | 0xXX | LSB | MSB |

Parameter explanation:

Scantime: one byte. It specifies the InventoryScan Time. Please also refer to 8.2.1.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x25 | 0x00 | - | LSB | MSB |

### 8.3.5 Set Baud Rate

The command is used to change the communication baud rate.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BaudRate |  |  |
| 0x05 | 0xXX | 0x28 | 0xXX | LSB | MSB |

Parameter explanation:

BaudRate: one byte. It specifies the communication baud rate as following table.

| BaudRate | Bps |
| --- | --- |
| 0 | 9600bps |
| 1 | 19200 bps |
| 2 | 38400 bps |
| 5 | 57600 bps (default) |
| 6 | 115200 bps |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x28 | 0x00 | - | LSB | MSB |

Notes: The response is still using the old baud rate. After this, later communication will use new baud rate.

### 8.3.6 Set RF Power

The command is used to adjust the RF output power of the reader.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Pwr |  |  |
| 0x05 | 0xXX | 0x2F | 0xXX | LSB | MSB |

Parameter explanation:

Pwr: one byte. It specifies the RF output power. The value range is from 0 to 30 with 30 for around 30dbm.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x2F | 0x00 | - | LSB | MSB |

### 8.3.7 Beep Setting

The command is used to set beep status.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BeepEn |  |  |
| 0x05 | 0xXX | 0x40 | 0xXX | LSB | MSB |

Parameter explanation:

BeepEn: one byte.Bit0=0:Close;Bit0=1:Open.

Bit1-Bit7: reserved.default is 0;

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x40 | 0x00 | - | LSB | MSB |

### 8.3.8 Set GPIO

The command is used to set the GPIO output TTL level. The default is TTL high level.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | OutputPin |  |  |
| 0x05 | 0xXX | 0x46 | 0xXX | LSB | MSB |

Parameter explanation:

OutputPin: one byte. It specifies the 2 output GPIOs’ (OUT1~OUT2) level. Bit0~Bit1 correspond to OUT1~OUT2.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x46 | 0x00 | - | LSB | MSB |

### 8.3.9 Get GPIO Status

The command is used to get the current 8 GPIOs output status.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x47 | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | OutputPin |  |  |
| 0x06 | 0xXX | 0x47 | 0x00 | 0xXX | LSB | MSB |

Parameter explanation:

OutputPin: one byte. It specifies the 2 output GPIOs’ (OUT1~OUT2) level and 2 input GPIOs’ (INT1~INT2) level. Bit4~Bit5 correspond to OUT1~OUT2,Bit0~Bit1 correspond to INT1~INT2.

### 8.3.10 Get Reader Serial number

The command is used to get Reader serial number.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x4C | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SerialNo |  |  |
| 0x09 | 0xXX | 0x4c | 0x00 | 4bytes | LSB | MSB |

Parameter explanation:

SerialNo: 4 bytes.Reader serial number.

### 8.3.11 Set Tag Custom function

The command is used to set tag custom function.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | InlayType |  |  |
| 0x05 | 0xXX | 0x3a | 0xXX | LSB | MSB |

Parameter explanation:

InlayType: tag Type,range is 0-254.

Default is 0,not specify type.

Value 1 is enabled Monza4QT Peek function.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | InlayType |  |  |
| 0x06 | 0xXX | 0x3a | 0x00 | 0xXX | LSB | MSB |

Parameter explanation:

InlayType: tag Type,range is 0-254.

### 8.3.12 Set Buffer EPC length

The command is used to set the EPC’s length of tag in the buffer.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SaveLen |  |  |
| 0x05 | 0xXX | 0x70 | 0xXX | LSB | MSB |

Parameter explanation:

SaveLen: one byte.EPC length.

0x00:128bits;

0x01:496bits;

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x70 | 0x00 | - | LSB | MSB |

### 8.3.13 Get Buffer EPC length

The command is used to get the EPC’s length of tag in the buffer.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x71 | - | LSB | MSB |

Parameter explanation:

None.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x06 | 0xXX | 0x71 | 0x00 | SaveLen | LSB | MSB |

SaveLen: one byte.EPC length.

0x00:128bits;

0x01:496bits;

### 8.3.14 Get Buffer data

The command is used to get data from buffer.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x72 | - | LSB | MSB |

Parameter explanation:

None.

Response:

| Len | Adr | reCmd | Status | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Num | EPC Data |  |  |
| 0xXX | 0xXX | 0x72 | 0xXX | 0xXX | EPC-1, EPC-2, … EPC-n | LSB | MSB |

Status:

| Value | Comment |
| --- | --- |
| 0x01 | Tag inventory operation is completely finished and all tags’ EPC data are returned. |
| 0x03 | The number of the inventoried tags is too much and not all tags’ EPC data can be returned within this single response data block. Other response data blocks are followed. |

Num: the tag number of this single response.

EPC Data: the EPC data in the buffer.

| EPC-n |  |  |  |  |
| --- | --- | --- | --- | --- |
| Ant | Len | EPC | RSSI | Count |
| 0xXX | 0xXX | nBytes | 0xXX | 0xXX |

Ant: It describes from which antenna the tag EPC is collected. For example, Ant = 0000 0100b means ANT3 and Ant = 0000 1000b means ANT4, etc.

Len :EPC length of this tag.

EPC: the tag’s EPC data.length is Len.

RSSI: the signal strength of  the tag for the first Read time

Count: The number of the successful reading tag, the value 0xFF said number greater than or equal to 255 times.

### 8.3.15 Clear Buffer

The command is used to clear tag data in the buffer.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x73 | - | LSB | MSB |

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x73 | 0x00 | - | LSB | MSB |

### 8.3.16 Get Buffer tag number.

The command is used to get tag number in the buffer.

Command:

| Len | Adr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x74 | - | LSB | MSB |

Parameter explanation:

None.

Response:

| Len | Adr | reCmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Count |  |  |
| 0x07 | 0xXX | 0x74 | 0x00 | 2bytes | LSB | MSB |

Count: 2 bytes.tag number in the buffer.