| 东集 EUIC S |  |  |  |
| --- | --- | --- | --- |
| Android UHF Interface User Manual |  |  |  |
| www.seuic.com |  |  |  |
| Revising index |  |  |  |
| Version | Update date | Revising details | By |
| 1.0 | 9/1/2016 | Create the Chinese version | Xiaodong Feng |
| 1.1 | 1/2/2019 | Increase the number of blockwrite and blockerase interfaces | Xiaodong Feng |

Content

Abstract	3rd

Hardware platform	3rd

Software platform	3rd

Object oriented	3rd

Quote method	3rd

UHF Interface	4th

UHFService class	4th

Using method	4th

Function interface	4th

1.	Switch UHF on	5th

2.	Switch UHF off	5th

3.	Get the firmware version number	5th

4.	Get the temperature	5th

5.	Get the power	5th

6.	Set the power	6th

7.	Get field	6th

8.	Set field	6th

9.	Singe tag reading	6th

10.	Start continuous tag reading	6th

11.	Stop start tag reading	7th

12.	Get the quantity of the continuous tags reading	7th

13.	Get the continuous tag reading ID	7th

14.	Read the label	7th

15.	Write the label	8th

16.	Block write the label	8th

17.	Block erase the label	8th

EPC class	9th

Class member variable	9th

Using method	9th

Function interface	9th

1.	Get the ID data from EPC oriented object	9th

2.	Match the ID number	9th

# Abstract

This document is created to provide the users the manual for the interface related to D500 device, to help them better use the special functions of the terminal product.

## Hardware platform

SDK is applicable to the devices listed below：

- D500

Without further limits definition in functional interface instruction, its using range shall be redeemed as applicable to all.

## Software platform

SDK is based on Android 4.4, and it supports the eclipse development tool.

## Object oriented

It’s designed for the developers who would like to use UHF module besides the Android standard functions.

## Quote method

- Take Eclipse for example, right click Program, then Properties, Java Build Path, Libraries, Add External JARs, select uhf.jar.

- AndroidManifest.xml uses uses-library to ID the uhf.jar is quoted package.

<application

……

<uses-library

android:name="com.seuic.uhf" />

……

</application>

# UHF Interface

| Package name | com.seuic.uhf |
| --- | --- |
| Package file | uhf.jar |
| System package | Yes |
| Class | UHFService |
| Function | Provide the control interface related to UHF |

## UHFService class

### Using method

import com.seuic.uhf.UHFService;

UHFService mDevice = UHFService.getInstance();

### Function interface

| Function | Specification |
| --- | --- |
| open | Switch UHF on |
| close | Switch UHF off |
| getFirmwareVersion | Get the firmware version number |
| getTemperature | Get the temperature |
| getPower | Get the power |
| setPower | Set the power |
| getRegion | Get field |
| setRegion | Set field |
| inventoryOnce | Singe tag reading |
| inventoryStart | Start continuous tag reading |
| inventoryStop | Stop continuous tag reading |
| getTagIDCount | Get the quantity of the continuous tags reading |
| getTagIDs | Get the continuous tag reading ID |
| readTagData | Read the label |
| writeTagData | Write the label |
| blockWriteTagData | Block write the label |
| blockEraseTagData | Block erase the label |

- Switch UHF on

boolean  open ()

Parameters

non

Return value

boolean; true for success and false for failure

- Switch UHF off

void  close ()

Parameters

Non

Return value

non

- Get the firmware version number

String  getFirmwareVersion ()

Parameters

non

Return value

String: not null for success and null for failure

- Get the temperature

String  getTemperature ()

Parameters

non

Return value

String: not null for success and null for failure  (℃)

- Get the power

int  getPower ()

Parameters

non

Return value

int; not-o for success and 0 for failure  (0~30 dBm)

- Set the power

boolean  setPower (int power)

Parameters

power

Set the power as (0~30 dBm)

Return value

boolean; true for success and false for failure

- Get field

String  getRegion ()

Parameters

non

Return value

String; non blank for success and null for failure

Range value including：”FCC” , ”ETSI”, ”China1” , ”China2”

- Set field

boolean  setRegion (String region)

Parameters

region

Range value including：”FCC” , ”ETSI”, ”China1” , ”China2”

Return value

boolean; true for success and false for failure

- Singe tag reading

boolean  inventoryOnce(EPC epc, int timeout)

Parameters

epc

EPC class oriented object

timeout

Order time out  (0~500 ms)

Return value

boolean; true for success and false for failure

- Start continuous tag reading

boolean  inventoryStart ()

Parameters

non

Return value

boolean; true for success and false for failure

- Stop start tag reading

boolean  inventoryStop ()

Parameters

non

Return value

boolean; true for success and false for failure

- Get the quantity of the continuous tags reading

int  getTagIDCount ()

Parameters

non

Return value

int; non-0 for the quantity of the labels got, and 0 for noting getting any label

- Get the continuous tag reading ID

List<EPC>  getTagIDs ()

Parameters

non

Return value

List<EPC>; The List collection of all EPC oriented objects

- Read the label

boolean  readTagData (byte[] Epc, byte[] PassWord, int Bank, int Offset ,int Len, byte[] Data)

Parameters

Epc

Label ID（PC+EPC）

PassWord

Visit password (length as 4 bytes)

Bank

Label saving area(0 for password,1 for EPC, 2 for TI and 3 for users)

Offset

Starting address (unit as byte)

Len

the length to read, with unit as byte

Data

save the data to be read in the label

Return value

boolean; true for success and false for failure

- Write the label

boolean  writeTagData (byte[] Epc, byte[] PassWord, int Bank, int Offset ,int Len, byte[] Data)

Parameters

Epc

Label ID（PC+EPC）

PassWord

Visit password (length as 4 bytes)

Bank

Label saving area(0 for password,1 for EPC, 2 for TI and 3 for users)

Offset

Starting address (unit as byte)

Len

the length to write, with unit as byte

Data

save the data to be written into the label

Return value

boolean; true for success and false for failure

- Block write the label

boolean  blockWriteTagData (byte[] Epc, byte[] PassWord, int Bank, int Offset ,int Len, byte[] Data)

Parameters

Epc

Label ID（PC+EPC）

PassWord

Visit password (length as 4 bytes)

Bank

Label saving area(0 for password,1 for EPC, 2 for TI and 3 for users)

Offset

Starting address (unit as byte)

Len

The length to write, with unit as byte

Data

save the data to be written into the label

Return value

boolean; true for success and false for failure

- Block erase the label

boolean  blockEraseTagData (byte[] Epc, byte[] PassWord, int Bank, int Offset ,int Len)

Parameters

Epc

Label ID（PC+EPC）

PassWord

Visit password (length as 4 bytes)

Bank

Label saving area(0 for password,1 for EPC, 2 for TI and 3 for users)

Offset

Starting address (unit as byte)

Len

The length to erase, with unit as byte

Return value

boolean; true for success and false for failure

## EPC class

### Class member variable

byte[] id;

int len;

int rssi;

int count;

### Using method

import com.seuic.uhf.EPC;

EPC epc = new EPC();

### Function interface

- Get the ID data from EPC oriented object

String  getId()

Parameters

non

Return value

String; a hexadecimal string

- Match the ID number

boolean equals(Object obj)

Parameters

EPC oriented object

Return value

boolean; true for success and false for failure